# Weight Station - Frontend UI Guide

## Overview

Weight Station is an industrial IoT application for monitoring load-cell sensors, counting manufactured pieces, and tracking weight-based production metrics.

Access the UI: Open a browser and navigate to http://<device-ip>:8080

# Navigation

The application has a sidebar with 6 pages:

| Page | Purpose |
| --- | --- |
| Dashboard | Overview with live charts and current readings |
| Sensors | Configure and monitor physical weight sensors |
| Counters | Configure counters that aggregate sensors and count pieces |
| Pieces | Define part types with expected weight and tolerance |
| WiFi | Manage wireless network connections |
| System | Hostname, config backup/restore, system info |

# Adding Sensors

Sensors must be configured before counters can use them.

Path: Sensors page

- Click the Add Sensor button

- Fill in the form:

- Sensor Name - Friendly name (e.g. “Main Scale”)

- Sensor Type - Choose Mock Sensor (Dev) for testing or HX711 Load Cell for real hardware

- Smoothing Samples - Number of readings to average (default: 5)

- Invert Sensor Reading - Check if the sensor reads upside-down

- Enable Sensor - Keep checked to activate

- Description - Optional notes

- If you chose Mock Sensor, you will see additional simulation fields:

- Simulation Mode: single (one item at a time) or multi (boxes with items)

- Initial Value: Starting weight in grams

- Step Value: Weight change per step

- Noise Range: Random noise added to readings

- Step Interval: Reads between weight changes

- If you chose HX711 Load Cell, you will see:

- Data Pin (DT) - GPIO pin number

- Clock Pin (SCK) - GPIO pin number

- Gain - 128, 64, or 32

- Value per Gram - Calibration factor

- Tare Offset - Raw offset for zeroing

- Click Save

The sensor appears in the table with a live value that updates every 500ms.

# Adding Pieces

Pieces define the parts you want to count. Each piece has an expected weight and tolerance.

Path: Pieces page

- Click the Add Piece button

- Fill in the form:

- Piece Name - Friendly name (e.g. “Steel Bracket”)

- SKU / Part Number - Optional part number (e.g. “BRK-001”)

- Expected Weight (g) - Target weight in grams

- Tolerance (±g) - Acceptable weight deviation

- Description - Optional notes

- Click Save

The Range column automatically shows Expected - Tolerance to Expected + Tolerance.

# Adding Counters

Counters aggregate one or more sensors into a virtual metric and detect when pieces are added or removed based on weight changes.

## Prerequisites

- At least one sensor must be configured (see Adding Sensors)

- Optionally, pieces should be configured (see Adding Pieces)

## Steps

- Click the Add Counter button

- Fill in the form:

| Field | What to put | Required |
| --- | --- | --- |
| Counter Name | Friendly name for this counter (e.g. “Line 1 Total”) | Yes |
| Sensors | Check the boxes for sensors to aggregate | Yes |
| Operation | How to combine multiple sensors: Sum, Average, Maximum, or Minimum | Yes |
| Check Interval (ms) | How often to poll sensors and check for changes (e.g. 1000 = 1 second) | No |
| Detection Threshold | Minimum weight change in grams to trigger a count event (e.g. 10) | No |
| Piece Selection API URL | External URL to fetch the active piece SKU (leave blank for manual) | No |
| Active Piece SKU | Manually set which piece type to track (overridden by API URL if set) | No |
| Enable Counter | Check to activate the counter | Yes |
| Description | Optional notes | No |

- Click Save

# Counter Table - Live Data

After adding a counter, the table shows:

| Column | Description |
| --- | --- |
| Name | Counter name |
| Sensors | Number of sensors attached |
| Active Piece | Currently tracked piece type (updates live) |
| Live Value | Current aggregated weight in grams |
| Counts | Running tally per piece type (e.g. P1: 5, P2: 3) |
| Status | Enabled or Disabled badge |
| Actions | Edit, Delete, and Clear buttons |

Data refreshes automatically every 500ms.

# Counter Actions

## Edit

Click Edit on any counter row to modify its settings. The modal opens pre-filled with current values.

## Delete

Click Delete to permanently remove the counter. You will be asked to confirm.

## Clear

Click Clear to reset the count to zero without deleting the counter configuration.

# How Counters Work

- The counter daemon reads all enabled sensors at the configured interval

- It maintains a rolling buffer of 7 samples per counter

- It compares the oldest 3 samples to the newest 3 to detect trends

- If the trend exceeds the Detection Threshold, it calculates how many pieces fit in the weight delta

- Pieces are counted when the weight change matches the expected weight ± tolerance of the active piece

# Dashboard

The Dashboard (/) provides a high-level view:

- Sensor Values chart - Real-time line chart of all sensor readings

- Counter Values chart - Real-time line chart of counter aggregated values

- Current Readings - Live sensor values in grams

- Counter Counts - Piece tallies per counter

All data refreshes every 500ms.

# WiFi Page

- Current Connection - Shows SSID, IP, and signal strength

- Saved Networks - List of known networks with a Forget button

- Scan - Click Scan Networks to find available WiFi networks

- Connect - Click Connect on a scanned network, enter the password, and save

# System Page

- Hostname - View and change the device hostname

- Network Info - IP address, MAC address, hostname

- WiFi Status - Current connection summary

- Config Export - Download all settings as a ZIP file

- Config Import - Upload a previously exported ZIP file to restore settings

- Version - Application version

# Data Files

All configuration is stored in JSON files in the config/ directory:

| File | Contents |
| --- | --- |
| sensors.json | Sensor definitions |
| pieces.json | Piece weight definitions |
| counters.json | Counter definitions |
| fields.json | Form field schemas for all three above |

Changes made through the UI are saved to these files automatically.

# Tips

- Start with Sensors, then Pieces, then Counters - this is the dependency order

- Use Mock Sensor type for testing without hardware

- The dashboard charts show the last 60 data points per sensor/counter

- Use Config Export before making bulk changes, so you can restore if needed